Signaling Plan File Norm

- Purpose: This document describes the use and structure of the Signaling Plan file.

- Use: The logic engine must receive a description of the signaling system of the area to analyze. This information is contained in the Signaling Diagram (SD), often in the form of AutoCAD file formats. Automatically translating the information in the SD to a distilled and structured format, suitable as input to the logic engine is not trivial. So until said translator is built, human intervention shall be depended on for the task of reading the SD and converting it’s information to a format suitable for admittance by the logic engine. For later ease of integration of the automatic reader, the logic engine will receive a JSON file containing relevant information. So when the reader is built, it will export said JSON file. But although JSON is human readable, the process of hand translating the signaling data can be expedited by the use of a intermediary file type, the Signaling Plan, taking advantage of the specificities of the application and workflow considerations. There will be a piece of software purposed with translating this file to the JSON file.

- Structure: One file can handle a full interlocking. Keys “STATION”, “SECTION”, “NODE”, “TRANSITS”, “SIGNAL” and “POINT”, as well as tabs and white spaces are used for structuring.

There is a hierarchical structure: “STATION” > “SECTION” > other keys.

Data is only introduced after each key.

IMPORTANT: the basic structural element is the section. Sections with points are like other sections, the point is a sub element of the section, despite the frequently equal naming. In the same way, signals are considered sub elements of sections.

Tabs are used to highlight the hierarchy previously described. Although their use is not necessary for machine readability, it is enforced to promote human readability.

White spaces are used to separate keys from their respective parameters, and different parameters from themselves when they are filled on the same line.

- Key description:

STATION: Station name. No white spaces.

SECTION: Section name. No white spaces. Corresponds to the previously specified station.

NODE: Node letter, according to the method described in Annex 1. After the letter is a white space, followed by the Kilometric Point (PK) of the node.

After another white space follows the optional connected section or block. This must be the same name used after the respective SECTION key. If no data is provided, a terminal node is assumed. After the last white space, follows the optional node letter of the connected section. If not provided, connection with block is assumed.

TRANSITS: Between white spaces, the possible transits between nodes of the section are provided by displaying the two respective node letters next to each other. Order matters.

SIGNAL: Only specified if the section contains signals. One key for each signal. After the key and a white space, follows the signal name. After another white space follows the signal’s PK. After another white space follows the signal direction, according to Annex 2. After the final white space follows the signal type, according to Annex 3.

POINT: Only specified if the section contains points. One key for each point. After the key and a white space, follows the point name. After another white space follows the point’s clearing point PK. After another white space follows the specific transits of the point (Annex 4), each separated by a white space.

Sample Signaling Plan for Bifurcação de Sapataria (BSP - OESTE):

STATION BSP

SECTION 428A

NODE A 43942 A1 B

NODE B 428000 MAL1 A

TRANSITS AB BA

SIGNAL S1 43934 ASC MD

SECTION 440D

NODE A 43942 D2 B

NODE B 428000 MAL2 A

TRANSITS AB BA

SIGNAL SC1 43934 ASC MD

SECTION A1

NODE A 44167 2 C

NODE B 43942 428A A

TRANSITS AB BA

SECTION D2

NODE A 44167 2 B

NODE B 43942 440D A

TRANSITS AB BA

SECTION 2

NODE A 44323 D1 B

NODE B 44167 D2 A

NODE C 44167 A1 A

TRANSITS AB BA AC CA

POINT 2 44182 AB BA

SECTION D1

NODE A 44475 445 B

NODE B 44323 2 A

TRANSITS AB BA

SECTION 445

NODE A 45084 PNE B

NODE B 44475 D1 A

TRANSITS AB BA

SIGNAL S2 44483 DESC MD

Annex 1 – Node nomenclature

To each node of an element is assigned a letter form the Latin Alphabet. This process follows the following steps:

- The diagram of the element is inscribed in a rectangle, with the nodes contacting the rectangle edges.

- The geometric center of the rectangle is set as the origin of a coordinate plane.

- A ray with terminus at the origin and passing through point (0;1) is negatively rotated about the origin. The first node intercepted is assigned letter “A”. Consecutive nodes intercepted are assigned the following letters, in alphabetical order.

Annex 2 – Signal direction

Signals which are the origin of itineraries in which the train will move in the positive PK direction are ascending, in the file coded “ASC”.

The contrary are descending signals, in the file coded “DESC”.

Annex 3 – Signal type

Signals that can be the origin of main itineraries are coded “M”.

Signals that can be the origin of DOS itineraries are coded “D”.

Signals that can be the origin of shunt itineraries are coded “S”.

The three letters can be combined without white spaces, if the signal is combined.

Shunt limit fixed signals are coded “SLI” (shunt limit indicator).

Annex 4 – Specific transits

The specific transits of a point are the transits through the section containing the respective point, which are only possible if the point is set in reverse.

Note 1:

Derailers are considered points.

Note 2:

There is no support for trailable points.